Subject: Guide to the TP170A & S7-200 Demonstration Program

This paper describes the TP170A demonstration program and how to set it up.   First, an overview of the demo screen hierarchy is shown here:

Utilities

Applications

Functions

Transfer/Off/On

Bakery

Manual Entries

Oil Well

Text

Check PLC I/O

Buttons

Machine Status

(PHOTO & PLC Comm.)

Exit Runtime

Graphics

Process Status

(PHOTO)

Adjust Contrast

Security

Screen Maint.

PumpControl

(PLC Comm)

In/Out Fields

Passwords

Field Devices

(PHOTO)

Library

Bar Graphs

Demo Help

Online demo tips

Messages

Screen’s with a star indicate that the online S7-200 PLC access is needed to see the all the functionality

on that screen.  Other screens can be demonstrated in both the offline and online mode.

Hardware:

- Any S7-200 PLC with new generation instructions (S7-224 used in PLC project). S7 PLC address is 2.

- SIMATIC TP170A.  TP170A address is 1.

Versions and special tips:

- ProTool project is created in version 5.2

- S7-200 project is created with MicroWin32 version 3.0.1.000.

NOTE:  The S7-224 PLC uses instructions specific to the new generation S7-200’s.  Therefore, the older

S7-200 PLC projects will not compile the PLC project without program modification.

Setup Procedure:

1) Load PLC project into S7-200 PLC (change PLC model if necessary)

3) Turn on TP170A. Press ‘Transfer’ button on the menu that appears. This places TP170A

into download mode.

4) Bring up ProTool and open the TP170A demo configuration.

5) Connect a null modem cable between the TP170A 1F1A port and the PC COM port (where ProTool is loaded).  Siemens transfer cable number is 6ES790-1BF00-0XA0.

6) In ProTool click on the Transfer/Download icon (can also select menu items, File->Download).

7) Connect a MPI or a DP cable between the PLC port and the TP170A  1F1B port.  Siemens MPI cable     part-number is:  901-0BF00-0AA0.

8) After several seconds, you see a “Startup Completed” message on the TP170A screen.

9) To see animation on application screen (oil pump) you must have PLC in RUN mode.

Demonstrate the TP170A:

The system message showing whether you are successfully connected to the PLC, is shown on the lower portion of the startup window of the TP170A.  There are four screen buttons on the startup menu of the TP170A:

- Applications:  This button takes you to a menu of two sample applications.  One is an oil well and another is a bakery.  There are buttons for each application that show samples of how an actual application might look.  It is designed to give viewers ideas about how they could use the TP170A to visualize their applications. Q0.0 is set to the START/STOP button on TP170A  screen for the Bakery machine status.

- Functions: This button takes you to a menu of various feature screens that show possible options that can be configured on a TP170A.  Once you select one of the buttons on this menu, you there will be ‘Next’ and ‘Previous’ buttons on each screen.  You can use the Next/Previous buttons to scroll through the various feature screens, or you can exit each screen and get to the function screens with the menu buttons.

- Utilities:  This button takes you to a screen that shows the built-in functions used for screen maintenance, contrast adjustment, downloading, system message display, and password processing. You can check whether the PLC connection to the TP170A device is working by going to the Utilities screen and pressing the ‘System Messages’ button.  Messages collect in this window like in a FIFO queue.  Unlike previous Siemens touch panels, the system messages and event messages do not pop up on the screen automatically.  Use the ‘Check PLC I/O’ screen to set Q outputs on the S7-200 PLC.

- Demo Help: This screen explains that the tags in the demonstration project consist of both internal (no PLC attachment) and PLC tags.  The event message screen and the oil well application screens have tags that require the PLC connection in order to see the animation and effect of the PLC values.  However, if no PLC is available, the demo project can be shown successfully without a PLC connection.  The Demo Help screen also explains that you should use contrast adjustment if photographs in the TP170 project are not displayed clearly.

Please see TP170A equipment manual for additional details about the TP170A device.

### Warning about using Windows NT and MicroWin

If you connect the PC MicroWin monitor, the PLC, and the TP170A on the same network, you must select ‘not the only master’ for the PC’s interface card setup (SETTING PC/PG INTERFACE) in control panel.  If you have WindowsNT, then the network nodes do not stay connected, and will result in communication errors.  This is an incompatibility problem with WindowsNT drivers and the S7-200 PLC’s.  If you use Windows95/98, you need to go to Control Panel, select ports, and make sure that the COM port used for PC to PLC communication is set to FIFO buffer.  This prevents S7-200 communication errors with Windows95/98.

If only a TP170A and a S7-200 PLC are connected on the network (MPI or DP) then select this option. (This is the default setting on the demo project.)

If the TP170A, the PLC, and the PC are connected on the network (MPI or DP) then do not select this option.

NOTE:

You will find this option in ProTool by selecting Controllers, then pressing the Parameters button.

WARNING!

Under no circumstances should these applications and programs be put into real world control applications. Siemens accepts no responsibility for the misuse or abuse of this warning.

Contact Ron Powers at 770.871.3940 if there are any problems encountered or there are any questions.

HMI Marketing